- gyro subsystem

  - The gyroscope block gives us the angular velocities about the x, y, and z axes read using the gyroscope on the Segway

  - The y and z velocities are connected to terminator blocks because only the x velocity is needed for balancing

  - The double block is used to convert the data type from the gyroscope to a double

  - The sum block is used to subtract a number from the gyroscope reading

  - The gyro bias input is used to eliminate any bias in the gyroscope because an existing bias will be constantly integrated and therefore cause steady-state error

  - An element-wise gain block is used to convert the output of the gyroscope into a more suitable form, which is radians/second

  - Alpha dot is the second output port from the subsystem and represents the angular velocity of the pendulum (rad/s)

  - A discrete time integrator is used to get the integral of alpha dot. We have to use a discrete integral because we do not have continuous states

  - Alpha is the angle of the pendulum and is the first output port from this subsystem (rad)

- encoder subsystem

  - The encoder block gives us the number of pulses (720 pulse resolution) measured from the rotating wheel read using the encoder on the Segway

  - The double block is used to convert the data type from the encoder to a double

  - The gain block is used to convert the output of the encoder into a more suitable form, which is radians

  - Beta is the position of the encoder and is the first output port of this subsystem (rad)

  - The discrete derivative is used to get the derivate of beta. We have to use a discrete derivative because we do not have continuous states

  - Beta dot is the second output port from the subsystem and represents the angular velocity of the encoder (rad/s)

- A sum block is used to add beta and alpha together to get theta, the angle of the wheel (rad)

- A sum block is used to add beta dot and alpha dot together to get theta dot, the angular velocity of the wheel (rad/s)

- A mux block is used to create a 2x1 state vector [theta; theta dot]

- A gain block is used to convert [theta; theta dot] to [x; x dot] with relationship x=-Rw*theta where Rw is the radius of the wheel

- A mux block is used to create 4x1 state vector [x; x dot; alpha; alpha dot]

- A matrix multiplication gain block is used to multiply the state vector by the LQR gain matrix

- An element-wise gain block is used for sign correction to match the coordinate system defined in the free body diagrams

- The motor driver block supplies voltage to the motor with saturation at 3.56 V